give me a structured path to learn zephyr rtos

Here is a realistic, structured learning path for Zephyr RTOS in 2026. This path assumes you already have basic embedded C knowledge and have worked with at least one microcontroller (bare-metal or with Arduino/CMSIS/etc.). If you're completely new to embedded development, spend 2–4 weeks refreshing MCU basics first.

Zephyr has a noticeably steeper learning curve than FreeRTOS or bare-metal → but once you pass the initial toolchain + Devicetree/Kconfig hurdle, productivity becomes very good.

### Phase 0 – Prerequisites (1–2 weeks if needed)

- Solid C programming (pointers, bitwise, structs, volatile, static inline…)

- Understanding of basic RTOS concepts: tasks/threads, context switch, priority inversion, semaphore/mutex/queue

- Comfort with make/CMake + command line

- Git basics

- One of: STM32, nRF52/nRF53, ESP32, Raspberry Pi Pico 2, or any board with good Zephyr support

### Phase 1 – Setup & First Blinky (1–3 weeks)

Goal: Get the toolchain running and build/flash real code.

- Read → Why Zephyr? (10–20 min overview pages)

  - https://zephyrproject.org/

  - https://docs.zephyrproject.org/latest/introduction/index.html

- Follow official Getting Started Guide very carefully

  - https://docs.zephyrproject.org/latest/develop/getting_started/index.html

  - Choose native_posix or qemu_cortex_m3 first (simulator — no hardware risk)

  - Then repeat with real hardware

- Build & understand these samples (in order):

  - hello_world

  - basic/blinky

  - basic/thread

  - basic/syscall

- Learn the three magic commands you will type 500+ times:

- Bash

- west build -p auto -b your_board samples/basic/blinky

- west flash

- west build -t menuconfig

Recommended first boards (very well supported in 2026):

- nRF52840 DK / nRF5340 DK

- STM32 Nucleo F401RE / STM32H7

- B_U585I_IOT02A

- Raspberry Pi Pico 2

- native_posix (simulation)

### Phase 2 – Core Zephyr Concepts (3–6 weeks)

Build small projects while learning the key subsystems.

Order of priority (most important first):

| Order | Topic | Why important? | Key Samples / Docs path | Mini-project suggestion |
| --- | --- | --- | --- | --- |
| 1 | Threads & scheduling | Core of RTOS | kernel/threads, scheduling | Two threads blinking different LEDs at different rates |
| 2 | Devicetree (.dts / .dtsi / overlays) | Hardware description — biggest mindset shift | devicetree/index.rst | Add button → control LED |
| 3 | Kconfig & menuconfig/prj.conf | Software configuration | application/config/index.rst | Enable/disable UART, I2C, sensors |
| 4 | Device Driver Model | How almost every peripheral is accessed | kernel/drivers/index.rst | GPIO driver → PWM driver |
| 5 | Interrupts & ISR → deferred work | Real-time safety | kernel/interrupt, kernel/workqueue | Button press in ISR → thread notification |
| 6 | Synchronization primitives | Avoid race conditions | kernel/synchronization | Producer-consumer with queue + semaphore |
| 7 | Sleeping, timeouts, k_timer | Time-based behavior | kernel/time | Periodic sensor read |
| 8 | Logging (with LOG_MODULE_REGISTER) | Debugging lifeline | services/logging | Structured logging + levels |

Do not skip Devicetree and Kconfig — they are the #1 reason people give up on Zephyr.

### Phase 3 – Practical Intermediate Skills (6–12 weeks)

Choose 2–3 areas according to your interest/product:

- Networking → sockets / MQTT / LwM2M / HTTP client

- Bluetooth → BLE peripheral / central / mesh (nRF boards shine here)

- File systems → LittleFS on external flash / internal flash

- USB device (CDC ACM, MSC, HID…)

- Power management → pm_device, system suspend

- Sensors & IIO subsystem

- Flash map & settings subsystem (persistent config)

- MCUBoot + DFU / SMP (over-the-air update)

- Shell subsystem (very useful for debugging)

Build 2–3 real-ish mini-products, for example:

- BLE environmental sensor (temp + humidity → phone app)

- Wi-Fi HTTP IoT button

- USB mass storage bootloader

- Low-power logger to LittleFS

### Phase 4 – Production & Advanced Topics (ongoing)

- Out-of-tree drivers & boards

- Custom west commands / modules

- CI with Twister (Zephyr's test framework)

- Static analysis + Coverity

- Security features (PSA, mbedTLS, etc.)

- SMP (multi-core) behavior

- Tracing / Percepio Tracealyzer integration

- Memfault / crash reporting integration

- Writing maintainable Zephyr applications (layers, modules, CMake best practices)

### Recommended Resources – 2026 edition

Must-have / Official

- https://docs.zephyrproject.org/latest/ (always use latest or main)

- Zephyr Discord (very active)

- GitHub Discussions → zephyrproject-rtos/zephyr

Tutorials & Courses

- maksimdrachov/zephyr-rtos-tutorial (GitHub) — excellent beginner-friendly walkthrough

- Nordic Developer Academy nRF Connect SDK courses (Zephyr-based)

- Beningo Embedded Group Zephyr workshops / YouTube

- Linux Foundation AC6401 course (if you can afford structured training)

YouTube Playlists / Talks

- "Zephyr RTOS Tutorials" playlist (various creators)

- Zephyr Developer Summit talks (2024–2025 videos still very relevant)

- "Real Time in the Real World – Scheduler Details" (deep scheduler understanding)

Communities

- Zephyr Discord

- Reddit r/embedded + r/ZephyrRTOS (smaller)

- Zephyr weekly calls & meetups (YouTube)

Start small, be patient with Devicetree/Kconfig pain (weeks 2–5 are usually the hardest), and build real things — that's when Zephyr starts to feel powerful.

https://interrupt.memfault.com/authors/lampacher/

Introduction to Zephyr Part 1: Getting Started - Installation and Blink | DigiKey

https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-1-getting-started-installation-and-blink

Introduction to Zephyr Part 2: CMake Tutorial | DigiKey

https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-2-cmake-tutorial

Introduction to Zephyr Part 3: Kconfig Tutorial | DigiKey

https://www .digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-3-kconfig-tutorial

Kconfig in Zephyr RTOS — detailed but structured explanation

Kconfig is the configuration system used to control which features, subsystems, drivers, stack sizes, logging levels, etc. are compiled into the firmware.

Zephyr borrowed it directly from the Linux kernel (same language, very similar semantics), but adapted for embedded RTOS use cases.

### Core purpose in Zephyr

Decide at build time:

- Which kernel features are included (threads? timers? workqueues? SMP? …)

- Which subsystems / libraries are built in (Bluetooth? USB? File systems? Shell? …)

- Which drivers are compiled (UART? I2C? SPI? GPIO? sensors? …)

- Tuning parameters (stack sizes, buffer counts, log levels, timeouts …)

- Debug / tracing / assertions level

→ Goal = produce the smallest possible binary that still has everything your application needs.

### How it works — big picture flow

Application prj.conf

↑           board/*_defconfig

│                 ↑

│           SoC / arch Kconfig fragments

│                 ↑

All Kconfig files ───┼───────►  Kconfig evaluation engine (kconfiglib)

(kernel, drivers,   │         (Python-based)

subsys, modules,   │

boards …)         │

▼

Final merged configuration

│

zephyr/.config     ← human-readable key=value file

│

▼

Generated headers:

zephyr/include/generated/autoconf.h

→ #define CONFIG_XXX 1

→ #define CONFIG_YYY "value"

│

C/C++ compiler

### Key concepts & symbol types

| Type | Example | Meaning / usage | Typical values |
| --- | --- | --- | --- |
| bool | CONFIG_GPIO | Yes/no — include this feature/driver | y / n |
| int | CONFIG_MAIN_STACK_SIZE | Numeric value (usually bytes, counts …) | 1024, 4096, etc. |
| hex | CONFIG_FLASH_BASE_ADDRESS | Hex number | 0x08000000 |
| string | CONFIG_KERNEL_BIN_NAME | Arbitrary text | "myfirmware" |
| choice | CONFIG_CONSOLE_BACKEND (one of) | Mutually exclusive options (radio buttons) | UART / RTT / USB_CDC_ACM |

Every symbol usually starts with CONFIG_

### Where symbols are defined

Kconfig files are scattered everywhere:

- kernel/Kconfig → scheduler, threads, timers, pipes, …

- subsys/Kconfig → logging, shell, settings, power management, …

- drivers/*/Kconfig → each driver family has its own file

- boards/*/Kconfig.board → board-specific defaults / selections

- soc/*/Kconfig.soc → SoC/family defaults

- arch/*/Kconfig → architecture options (ARM, RISC-V, …)

- modules (e.g. MCUboot, TF-M) → their own Kconfig files

### How you (application developer) normally change config

Most common ways — ordered by frequency:

- prj.conf (project configuration fragment)
→ create this file in your application folder
→ simplest & most recommended method

- kconfig

- # Enable Bluetooth CONFIG_BT=y CONFIG_BT_PERIPHERAL=y CONFIG_BT_DEVICE_NAME="My Zephyr Device" # Reduce RAM usage CONFIG_MAIN_STACK_SIZE=1024 CONFIG_IDLE_STACK_SIZE=512 # More debug output CONFIG_LOG=y CONFIG_LOG_MODE_IMMEDIATE=y CONFIG_LOG_DEFAULT_LEVEL=4

- menuconfig (terminal curses UI)
west build -t menuconfig
→ explore all options interactively, search with /, get help with ?

- guiconfig (Tk-based GUI)
west build -t guiconfig

- Directly edit zephyr/.config (not recommended for permanent changes)

- Command line fragments (advanced / CI)
west build -DCONF_FILE="prj.conf debug.conf release.conf"

### Important differences vs Linux Kconfig

| Aspect | Linux Kernel | Zephyr RTOS |
| --- | --- | --- |
| Binary size sensitivity | Important but less critical | Extremely critical (kb matter) |
| Default values | Often quite feature-rich | Very minimal by default |
| Dependencies | Complex, many hidden | Also complex, but stricter minimalism |
| Interactive tools | menuconfig, xconfig, gconfig | menuconfig + guiconfig (no xconfig) |
| Python lib | Original (old) | Modern kconfiglib (Python, very clean) |
| Fragments / defconfig | Used in some distros | Core workflow (prj.conf + board defconfig) |
| Hardware description | Partially in Kconfig | Mostly moved to Devicetree |

### Quick mental model — Kconfig vs Devicetree in Zephyr

- Kconfig → software / feature selection "Should we even compile the I2C driver at all?"

- Devicetree → hardware description & instance config "There is an I2C controller at 0x40005400 with SDA on pin PB7 …"

Both talk to each other (Kconfig can read devicetree properties via functions like dt_nodelabel_status()).

### Summary — one-liner cheat sheet

Kconfig = compile-time feature switchboard of Zephyr
→ controls what code gets built
→ lives in hundreds of Kconfig files
→ merged result → autoconf.h → #ifdef CONFIG_BT …
→ you normally tune it via prj.conf + menuconfig

Let me know if you'd like examples for specific subsystems (logging, Bluetooth, sensors, power management, etc.)!

=======================================================================

Introduction to Zephyr Part 4: Devicetree Tutorial | DigiKey

https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-4-devicetree-tutorial

refer : devicetree-specification-v0.4.pdf

https://docs.zephyrproject.org/latest/build/dts/intro-syntax-structure.html

A devicetree is a hierarchical data structure primarily used to describe hardware.

Device tree ->  A standardized format to describe system hardware in a hierarchical, tree-like structure

Zephyr uses devicetree in two main ways:

- to describe hardware to the Device Driver Model

- to provide that hardware’s initial configuration

Linux kernel uses Devicetree to load drivers and modules at runtime (during boot), Zephyr evaluates it at compile time, ensuring only relevant device drivers and configurations get compiled into your final firmware.

### Why Devicetree?

Portability and Abstraction: By describing hardware in Devicetree rather than code, you can reuse the same driver or application logic across different boards.

Clarity and Organization: Devicetree files provide a structured view of your hardware.

### The Devicetree Specification and Core Concepts

The official Devicetree specification is available at devicetree.org. It describes the format, reserved properties, and principles behind representing hardware as a hierarchical tree structure.

### Key Concepts

- Nodes: Each hardware component or subsystem is represented as a node.

- Tree Structure: Every node, except the root node /, has exactly one parent, forming a hierarchy.

- Properties: Nodes contain properties that define addresses, interrupts, compatibility strings, and other configuration details.

Some key properties you should be aware of:

- compatible: A crucial property that helps identify which driver should handle that node’s hardware.

- status: Indicates whether a device is enabled ("okay") or disabled ("disabled").

- reg and interrupts: Standard properties to specify memory-mapped addresses and interrupt lines for the device.

For example, a CPU node might be at /cpus/cpu@0. Another node might be /soc/gpio@4000 representing a GPIO controller at memory address 0x4000.

Phandles are references that the Devicetree compiler assigns to nodes so other nodes can reference them. This allows linking related hardware components together without duplication.

## .dts — the base device tree

Think of a .dts file as the full hardware description.

- Describes the entire board: CPU, RAM, peripherals, pins, clocks, etc.

- Usually board-specific

- Lives in the board definition directory

- Normally you don’t edit this for applications

Example:  boards/arm/nrf52840dk_nrf52840/nrf52840dk_nrf52840.dts

## .overlay — modifications on top of the base

A .overlay file is a patch applied on top of the board’s .dts.

- Enables/disables peripherals

- Changes pin assignments

- Adds sensors, LEDs, displays, etc.

- App-specific

- Safe and recommended to edit

Typical location: app.overlay  or    boards/nrf52840dk_nrf52840.overlay

### Devicetree Syntax Basics

/dts-v1/;

/ {

compatible = "my,board";

a-node {

a-sub-node: a-sub-node {

some-property = <0x1234>;

};

};

};

Some key points:

- The top-level line often contains "/dts-v1/"; indicating Devicetree version 1 format.

- The root node is /.

- Properties are key-value pairs inside a node’s braces.

- Labels (like subnode_nodelabel: a-sub-node { };) allow you to reference nodes by a simpler name rather than a full path.

### Addressing and Properties

### Aliases and Chosen Nodes

### Exploring Devicetree Files in Zephyr

Zephyr projects often have multiple DTS and DTSI (Devicetree Source Include) files. The .dtsi files are included by .dts files, much like C headers, providing a way to share common configurations (like SoC definitions or board-level specifics).

For instance, the ESP32-S3 DevKitC board’s configuration might involve several layers:

- Board DTS Files: zephyr/tree/main/boards/espressif/esp32s3_devkitc/esp32s3_devkitc_procpu.dts describes the hardware connected to the primary CPU.

- SoC DTSI Files: Includes like zephyr/dts/xtensa/espressif/esp32s3/esp32s3_common.dtsi contain definitions for peripherals common to all ESP32-S3 modules.

- Module DTSI Files: Board variants might include a specific WROOM module DTSI

In zehlyr application device tree is implemented using overlay file,Devicetree files (.dts or .overlay) describe hardware in a structured way.
Example: boards/esp32s3_devkitc.overlay

Introduction to Zephyr Part 5: Devicetree Bindings | DigiKey

https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-5-devicetree-bindings

## What exactly is a DeviceTree binding?

A DeviceTree binding in Zephyr is a YAML file that:

- Defines which properties a DeviceTree node may have

- Specifies property types (int, string, boolean, phandle, array, etc.)

- Marks properties as required or optional

- Describes compatible strings

- Documents the hardware interface for drivers

Location:

zephyr/dts/bindings/

File example:

zephyr/dts/bindings/gpio/gpio-controller.yaml

Bindings are the link between this abstract hardware description and the low-level driver code. They are typically defined in YAML files placed in the zephyr/dts/bindings directory. Each binding file corresponds to one or more compatible strings. A compatible string might look like "espressif,esp32-adc". Inside the corresponding binding file, you’ll find documentation on what properties that node can have, their types, default values, and what’s required vs. optional.

When you run west build, Zephyr’s build system uses these binding files to:

- Validate your DTS and overlay files.

- Generate C header files that map Devicetree nodes, properties, and configurations into macros and structs that driver code and your application can reference.

As a result, changing something in your DTS configuration automatically propagates to your code through these auto-generated headers and macros. This keeps your code clean, maintainable, and easy to adapt when you switch boards or peripherals.

Note that bindings files are unique to Zephyr—Linux does not use them to map Devicetree nodes to driver code. Zephyr reads them at compile time to enforce node properties in the Devicetree (interface) and to determine which driver files to use.

### Finding the Binding Files

You’ll find the ADC binding files in zephyr/dts/bindings/adc/. There should be a generic ADC binding (such as adc-controller.yaml) and a SoC-specific binding (espressif,esp32-adc.yaml), among others.

- adc-controller.yaml: Defines the standard properties for an ADC controller in Zephyr. Includes properties like #io-channel-cells and standard properties for defining channels. This file creates a common interface for ADC configurations.

- espressif,esp32-adc.yaml: Extends the generic ADC binding with properties specific to the ESP32 series.

These YAML files use a schema that lists required and optional properties, their types, and how the driver will interpret them.

Some key properties defined by these bindings include:

- reg: Indicates the memory address or bus address of the ADC peripheral.

- status: Should be "okay" to enable the ADC, or "disabled" otherwise. Note that Linux has other status values, but Zephyr sticks to these two (for now).

- zephyr,gain, zephyr,reference, zephyr,vref-mv, and zephyr,resolution: Define how the ADC should interpret input signals and what reference voltage or resolution to use. These are per-channel properties when you create channel sub-nodes.

Introduction to Zephyr Part 6: How to Write a Device Driver | DigiKey

https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-6-device-driver-development

- Introduction to Zephyr Part 7: Debugging with OpenOCD and GDB | DigiKey

- https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-7-debugging-with-openocd-and-gdb

- Introduction to Zephyr Part 8: Multithreading | DigiKey

- https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-8-multithreading

- Introduction to Zephyr Part 9: Interrupts, Timers, and Counters | DigiKey

- https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-9-interrupts-and-workqueues

- Introduction to Zephyr Part 10: Graphics with LVGL and Display Drivers | DigiKey

- https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-10-graphics-with-lvgl-and-display-drivers

- Introduction to Zephyr Part 11: WiFi and IoT | DigiKey

- https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-11-wifi-and-iot

- Introduction to Zephyr Part 12: How to Create a Custom Board Definition | DigiKey

- https://www.digikey.com/en/maker/tutorials/2025/introduction-to-zephyr-part-12-how-to-create-a-custom-board-definition

### Fastest & Most Efficient Path (3-6 Months to Solid Proficiency)

- Core principle: Learn by doing — build, break, debug daily on a dev board. Avoid passive reading; code every concept.

- Recommended hardware — Start with a well-supported board like Nordic nRF52840 DK, nRF5340 DK, STM32 Nucleo, or NXP FRDM series for easy flashing and examples.

- Tools — Use VS Code with Zephyr extensions (or West CLI) for faster workflow.

- Time estimate — 1-2 weeks setup + basics, 1-2 months core kernel/drivers, 2-4 months advanced + projects.

### Step-by-Step Roadmap

- Week 1: Setup & First Blink (Foundation)

  - Follow the official Zephyr Getting Started Guide — Install west, Python venv, Zephyr SDK, clone repos.

  - Build and flash the "blinky" sample on your board.

  - Use QEMU (emulation) if no hardware yet for quick tests.

  - Goal: Understand west build/flash, basic project structure.

- Weeks 2-4: Core Concepts (Kernel Basics)

  - Study RTOS fundamentals if new (threads, scheduling, interrupts, mutexes, timers).

  - Build/run samples: hello_world, threads, synchronization primitives.

  - Learn key Zephyr tools:

    - Kconfig (prj.conf for feature toggles)

    - Devicetree (.dts, overlays for hardware config)

    - Logging, shell for debugging.

  - Resource: Official samples + GitHub tutorial (maksimdrachov/zephyr-rtos-tutorial) — progressive lessons with exercises on threads → GPIO → scheduling → interrupts → timers → mutexes.

- Weeks 5-8: Drivers, Peripherals & Intermediate Skills

  - Explore drivers (GPIO, UART, I2C, SPI, ADC, PWM).

  - Use Nordic Academy courses (free) — nRF Connect SDK Fundamentals (covers Zephyr basics) + Intermediate (thread management, data passing).

  - Build projects: Sensor reading, UART console, simple BLE if on Nordic.

  - Practice overlays to customize boards without forking.

- Months 2-4: Advanced & Real-World Mastery

  - Dive into subsystems: Networking (TCP/IP, MQTT), File systems, USB, BLE.

  - Use tracing/logs/shell for observability.

  - Build a personal project: e.g., IoT sensor node, weather station, or multi-threaded app.

  - Debug with GDB, SEGGER tools, or Zephyr shell.

  - Read kernel docs + source code for deeper understanding.

- Ongoing: Best Practices & Community

  - Join Zephyr Discord/mailing list for questions.

  - Follow updates (west update regularly).

  - Contribute small fixes or drivers to GitHub for expertise.

  - Optional paid course: Linux Foundation's Zephyr RTOS Programming (hands-on labs).

  - Free video series: DigiKey's Intro to Zephyr (12 parts) or Mastering Zephyr RTOS (on-demand sessions on build systems, threads, drivers).

### Tips for Speed & Efficiency

- Use a single board initially — switch later for portability.

- Always use -p auto/pristine in west build to avoid config issues.

- Prioritize understanding Devicetree + Kconfig — they make Zephyr powerful/portable.

- Practice daily — 1-2 hours coding > binge reading.

- If stuck: Search docs first, then community (Zephyr wins over older RTOS like FreeRTOS for modern, scalable apps).

This hands-on, official-first approach is the fastest path — many developers report proficiency in months by building real apps early. Start today with the official guide!

# Nordic Academy Zephyr notes

Example code repo: https://github.com/NordicDeveloperAcademy/ncs-fund.git

nRF52833 DK

DK devicetree file

These hardware details are all described in the devicetree file for the nRF52833 DK.

<install_path>\zephyr\boards\nordic\nrf52833dk\nrf52833dk_nrf52833.dts

GPIO API: https://docs.zephyrproject.org/latest/doxygen/html/group__gpio__interface.html

### Core Idea

Use generic GPIO API (<zephyr/drivers/gpio.h>) to control LEDs (output) and read buttons (input) in a hardware-agnostic way via devicetree.

### Key Steps to Use GPIO

- Get pin spec from devicetree

- Check if device is ready

- Configure pin (input/output + flags)

- Read (input) or write/toggle (output)

### Main Structure: gpio_dt_spec

struct gpio_dt_spec my_pin;

Filled using:

GPIO_DT_SPEC_GET(DT_ALIAS(led0), gpios, &my_pin);     // or DT_NODELABEL(...)

Always check readiness:

if (!device_is_ready(my_pin.port)) { /* error */ }

### Most Important Functions

| Function | Purpose | Example Usage |
| --- | --- | --- |
| GPIO_DT_SPEC_GET(...) | Get pin spec from DT | GPIO_DT_SPEC_GET(DT_ALIAS(led0), gpios, &led) |
| gpio_is_ready_dt() | Check device ready | gpio_is_ready_dt(&led) |
| gpio_pin_configure_dt() | Set direction + flags | gpio_pin_configure_dt(&led, GPIO_OUTPUT ¦ GPIO_ACTIVE_LOW) |
| gpio_pin_set_dt() | Set output value (0/1) | gpio_pin_set_dt(&led, 1) → active |
| gpio_pin_toggle_dt() | Flip output state | gpio_pin_toggle_dt(&led) |
| gpio_pin_get_dt() | Read input state | int val = gpio_pin_get_dt(&button) |
| gpio_pin_interrupt_configure_dt() | Enable interrupt on input | GPIO_INT_EDGE_TO_ACTIVE (press detect) |
| gpio_init_callback() | Prepare callback struct | gpio_init_callback(&cb, isr_func, BIT(pin)) |
| gpio_add_callback() | Register ISR | gpio_add_callback(port, &cb) |

### LED (Output) – Typical Pattern

struct gpio_dt_spec led = GPIO_DT_SPEC_GET(DT_ALIAS(led0), gpios);

gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE);  // or | GPIO_ACTIVE_LOW

gpio_pin_toggle_dt(&led);                         // Blink

### Button (Input) – Two Methods

1. Polling (simple, CPU-heavy)

int state = gpio_pin_get_dt(&button);

if (state == 0) { /* pressed – if active low */ }

2. Interrupt (recommended – low power)

// Configure

gpio_pin_configure_dt(&button, GPIO_INPUT | GPIO_PULL_UP);

gpio_pin_interrupt_configure_dt(&button, GPIO_INT_EDGE_TO_ACTIVE);

// Callback

void button_isr(const struct device *dev, struct gpio_callback *cb, gpio_port_pins_t pins) {

gpio_pin_toggle_dt(&led);

}

static struct gpio_callback button_cb;

gpio_init_callback(&button_cb, button_isr, BIT(button.pin));

gpio_add_callback(button.port, &button_cb);

### Important Flags

- GPIO_OUTPUT / GPIO_INPUT

- GPIO_ACTIVE_LOW – active = logic 0

- GPIO_PULL_UP / GPIO_PULL_DOWN

- Interrupt: GPIO_INT_EDGE_TO_ACTIVE, GPIO_INT_EDGE_TO_INACTIVE

## Lesson 3 – Elements of an nRF Connect SDK application

## Lesson 4 – Printing messages to console and logging

header file <zephyr/sys/printk.h>

Printk() - known as synchronous logging, in-place logging, or blocking logging. Logs are sent immediately as they are issued, and printk() will not return until all bytes of the message are sent

Logger module

The logger module is the recommended method for sending messages to a console, unlike the printk() function, which will not return until all bytes of the message are sent. The logger module supports both in-place and deferred logging among many other advanced features such as:

- Multiple backends

- Compile time filtering on module level

- Run time filtering independent for each backend

- Timestamping with user-provided function

- Dedicated API for dumping data

- Coloring of logs

- printk() support – printk message can be redirected to the logger

LOG_X for standard printf-like messages, where X can be DBG, INF, WRN, or ERR.

Example:

LOG_INF("Exercise %d",2);

LOG_DBG("A log message in debug level");

LOG_WRN("A log message in warning level!");

LOG_ERR("A log message in Error level!");

Dumping data

We also have the LOG_HEXDUMP_X  macros for dumping data where X can be DBG, INF, WRN. LOG_HEXDUMP_INF(data, sizeof(data),"Sample Data!");  or ERR.

| Feature | printk() | Logger (Zephyr Logging Subsystem) |
| --- | --- | --- |
| Header | <zephyr/sys/printk.h> | <zephyr/logging/log.h> |
| Basic usage | printk("Value: %d\n", x); | LOG_INF("Value: %d", x); |
| When output happens | Immediately (synchronous) | Usually deferred (queued → separate thread) |
| Thread safety | Safe, but blocks caller (critical sections) | Non-blocking in deferred mode (recommended) |
| Message interleaving | Can garble in multi-thread / ISR if not careful | Handled safely by logging backend |
| Timestamp | No (must add manually) | Automatic (optional, configurable) |
| Module / Source info | No | Yes — module name, file, function, line |
| Log levels | None (always prints) | Yes: ERR, WRN, INF, DBG (per module + global filtering) |
| Runtime filtering | No | Yes — enable/disable levels per module at runtime/build |
| Memory / overhead | Very low | Higher (buffers, thread, formatting) — but very configurable |
| Output control | Direct to console backend | Through backends (UART, RTT, network, custom, etc.) |
| printk → logging bridge | — | Enable CONFIG_LOG_PRINTK=y → redirects printk to logger |
| Best for | Bootloader, early bring-up, very low-level debug, minimal footprint | Normal application debugging, production logging, multi-thread safety |
| Typical Kconfig | CONFIG_PRINTK=y | CONFIG_LOG=y + CONFIG_LOG_MODE_DEFERRED=y (default now) + level selection |

## Lesson 5 – Serial communication (UART)

Zephyr's generic I2C API (<zephyr/drivers/i2c.h>) makes I2C communication hardware-agnostic and portable using devicetree.
Use struct i2c_dt_spec to bundle the I2C bus controller + slave address.

### Enable I2C Driver

In prj.conf:

CONFIG_I2C=y

Include:

#include <zephyr/drivers/i2c.h>

### Key Structure: i2c_dt_spec

struct i2c_dt_spec {

const struct device *bus;   // I2C controller device

uint16_t addr;              // 7-bit or 10-bit slave address

};

Get it from devicetree:

#define MY_SENSOR_NODE DT_NODELABEL(my_sensor)   // or DT_ALIAS(i2c_sensor)

static const struct i2c_dt_spec sensor = I2C_DT_SPEC_GET(MY_SENSOR_NODE);

Always check:

if (!device_is_ready(sensor.bus)) {

printk("I2C bus not ready!\n");

return -ENODEV;

}

### Devicetree Setup (Most Important Step!)

Add/overlay your I2C peripheral as a child node under the I2C controller.

Example overlay (boards/nrf52840dk_nrf52840.overlay):

dts

&i2c0 {                     // or &i2c1, &i2c2 depending on board/pins

status = "okay";

mysensor@48 {           // @ = slave address in hex

compatible = "mycompany,mysensor";  // dummy or real driver compatible

reg = <0x48>;       // 7-bit I2C address

label = "mysensor";

};

};

Then use DT_NODELABEL(mysensor) in code.

### Main API Functions (DT variants – preferred)

| Function | Purpose | Return | Typical Use Case |
| --- | --- | --- | --- |
| i2c_write_dt() | Write bytes to device | 0 or <0 error | Set register / config |
| i2c_read_dt() | Read bytes (after setting register if needed) | 0 or <0 | Raw read after write |
| i2c_reg_read_byte_dt() | Read 1 byte from a register | 0 or <0 | Quick single-reg read |
| i2c_burst_read_dt() | Read multiple bytes starting from reg | 0 or <0 | Read sensor data block |
| i2c_write_read_dt() | Write then immediately read (combined) | 0 or <0 | Most common pattern: set reg → read value |

All take const struct i2c_dt_spec *spec as first arg.

### Code Patterns (Most Frequent)

- Write single register

- uint8_t reg   = 0x01;     // register address

- uint8_t value = 0x80;     // data to write

- i2c_write_dt(&sensor, &reg,   1);

- i2c_write_dt(&sensor, &value, 1);

- Read single register (convenience)

- uint8_t reg_addr = 0x0F;  // WHO_AM_I register example

- uint8_t who_am_i;

- int ret = i2c_reg_read_byte_dt(&sensor, reg_addr, &who_am_i);

- if (ret == 0) {

- printk("WHO_AM_I = 0x%02x\n", who_am_i);

- }

- Most common: Write register pointer → Read data

- uint8_t start_reg = 0x28;   // e.g. accel data start

- uint8_t buf[6];             // 3 axes × 2 bytes

- ret = i2c_write_read_dt(&sensor, &start_reg, 1, buf, sizeof(buf));

- Burst read from start register

- i2c_burst_read_dt(&sensor, 0x28, buf, 6);

### Error Handling & Tips

- Every function returns 0 on success, negative errno on failure (-EIO, -ENXIO, -EINVAL, -ETIMEDOUT, etc.)

- Always check return values!

- if (ret < 0) {

- printk("I2C error: %d\n", ret);

- }

- Pull-ups usually required on SDA/SCL (board-dependent; Nordic DKs often have them).

- Use i2c_write_read_dt() whenever possible → atomic, avoids race conditions.

- 10-bit addressing: rare, but addr supports it (set high bit if needed).  For multi-byte transfers → prepare buffer correctly (MSB first usually).

### Quick Checklist for New Sensor

- Find I2C address (datasheet).

- Add overlay with correct reg and label.

- Get i2c_dt_spec with I2C_DT_SPEC_GET(DT_NODELABEL(...)).

- Check device_is_ready().

- Use i2c_write_read_dt() for typical read sequences. Handle errors and log.

## Lesson 7 – Multithreaded applications

Kernel APIs: https://docs.zephyrproject.org/latest/doxygen/html/group__kernel__apis.html

Having the main() function is optional in Zephyr RTOS-based applications. This is because the main thread automatically spawned by the RTOS will do the necessary RTOS initialization, including scheduler/kernel setup, and core drivers setup.

After that, it will try to call the user-defined main(), if one exists. If no main() function exists, the main thread will exit. The system will still be functional and other threads can be scheduled normally.

=================================================================================

# nRF Connect SDK Intermediate

## Lesson 1 – Zephyr RTOS: Beyond the basics